DEPARTMENT OF THE AIR FORCE

SHADOW MOSES AIR FIELD, ALASKA

RAPID ASBESTOS SURVEY RESULTS

Campbell Hall interior wall systems | Report SM-PLM-27-114

| Client | 713th Civil Engineer Squadron (713 CES) |
| --- | --- |
| Collected | 7–8 July 2027 by Dr. Para-Medic, CIH |
| Analyzed | 9 July 2027 by polarized-light microscopy with dispersion staining |
| Scope | Representative concealed joint compound and backing from 20 interior wall-system locations |

# Summary by building zone

| Zone | Wall systems sampled | Systems with ACM | Observed asbestos content |
| --- | --- | --- | --- |
| North administration | 7 | 7 | 3–7% chrysotile in joint compound / backing |
| Secure briefing core | 4 | 4 | 4–8% chrysotile in joint compound / backing |
| East office corridor | 4 | 4 | 3–6% chrysotile in joint compound / backing |
| West service wing | 5 | 4 | 2–7% chrysotile in four systems; one negative |
| TOTAL | 20 | 19 | 19 of 20 representative wall systems positive |

# Laboratory conclusion

Nineteen of the twenty representative wall systems contain asbestos-bearing concealed finish or backing material. On the sampled record, asbestos-containing material is present in approximately ninety-five percent of Campbell Hall's interior wall-system types and is not limited to the utility chases identified in the historical record.

WORK RESTRICTION. Do not cut, drill, sand, demolish, or remove affected wall assemblies until an approved abatement method, regulated work area, clearance plan, and qualified personnel are in place.

/s/ PARA-MEDIC
Dr. Para-Medic, CIH | Laboratory Technical Director
9 July 2027